SBE Processing configurations used for EN410 & 414 Kincaid:

# CTD001-015 (All EN410 casts) CTD020-027 (All EN414 casts) have WET Labs ECO Fluorometer, Transmissometer, Biopherical PAR, Benthos Altimeter, Oxygen1, Oxygen 2 and Biopherical Surface PAR. Temperature and Conductivity sensors were changed prior to EN414 to more recently calibrated units.

# 425Datcnv_a

X Process scans to end of file. Scans to skip over 0. Binary output, Up& downcast, Create converted data (.CNV) file only. X Merge separate header file.

Scan Count, Time Elapsed (s), Pressure Digiquartz (db), Depth seawater (m), Temp 1, Temp 2, Temp Diff, Cond 1, Cond 2, Cond Diff, V0, Beam Attenuation Seatech, Beam Transmission Seatech, V2, Altimeter, V3, PAR, V4, Oxygen V SBE43 1, V5, Oxygen V SBE43 2, V7, Fluorescence WETLab ECOFL (mg/m^3), Pump Status, Latitude, Longitude.

425Filter_a

“FILTER can be run before CELLTM to remove any residual response time between the temperature and conductivity sensors and to minimize digitization noise.  On average, a low pass filter with a time constant of 0.03 sec reduces the noise in computed salinity slightly.”

for SBE9/11 SBE9plus, older manualSBE4249.txt suggests

Low pass filter B = 0.15s for pressure

Low pass filter A = 0.03s for conductivity

New SBEDataProcessing_5.32a.pdf suggests - for conductivity

# Used: B=0.15 for Press, Depth. A=0.03 for Cond 1 & 2 & Diff

# 425AlignCTD_a

# Cond 0.073s (secondary only)

# Temp 0

# DO 4sec (2 at 25C, 5 at 0C)   V4, Oxy V pri, V5, OxyV

425CellTM_a for 9+ w/TC duct & pump running at 3000rpm.

Correct both pri & sec conductivity values

alpha = 0.03

1/beta = 7

# LoopEdit (not done for EN425)

Fixed minimum velocity of 0.25m/s. X Exclude scans marked bad.

# 425Derive_a

Depth (sw, m 43.35Deg N), Salinity 1 & 2 (PSU), Density 1 & 2 (sigma-t, Kg/m^3), Descent Rate (m/s), Oxygen 1 & 2, Pot Temp 1 & 2 (deg C), Sound Velocity 1 & 2 (Chen-Millero, m/s)

# 410BinAvg_a

Pressure, 0.1, X Incl #scans/bin, X exclude scans marked bad, skip over 0, Process Downcast only, X Incl surface bin 0, 0, 0

# 425ASCII_Out_a

# Output header & data files, 60 lines/pg, Label columns top of file, Semicolon, Julian days, all variables.