CTD SYSTEM CONFIGURATION			Cruise:	EN425		Dates:	24 Aug - 10 September 2005

Tech: Lynne Butler, Dave Nelson				Chief Scientist, Institution: Jim Lynch, WHOI

Program, versions					SeaSave Win32 v 5.37d, DOS Seasoft 4.249, SBE Processing v5.32a

File names						425f_0607*.cfg

Batch files used	pro425_a*.bat to process, copy to archive

Deck cal files						EN425\CTD\sswin\CalData

SBE 9plus CTD underwater unit (6800m depth capability)	S/N	12293-0444 (07-Mar-03), 23539-0607, none

Pressure sensor, Digiquartz (with temp comp, SBE)		S/N	64853 (07-Mar-03), 80016 (03-Mar-06)

SBE 11plus deck unit					S/N	11P12293- 0426, 0427 (21-Mar-97), none

Temperature sensor, primary (3400 m, 6800m, slow, fast)	S/N	604, 753, 1164, 1185, 1678,

(SBE 3, 3Plus)				2034, 2107 (15-Feb-06), 2899, 2902, 4126, 4130, 4333

Temperature sensor, secondary (3400 m, 6800m, slow, fast)	S/N	604, 753, 1164, 1185, 1678,

(SBE 3, 3Plus)				2034, 2107, 2899, 2902 (10-Sep-05), 4126, 4130, 4333, none

Conductivity sensor, primary (SBE 4C, 3400m, 6800m)	S/N	199, 200, 618, 856, 864, 1446,

1578, 1745, 1749, 2459, 2469 (08-Mar-06), 2822

Conductivity sensor, secondary (SBE 4C, 3400m, 6800m)	S/N	199, 200, 618, 856, 864 (09-Feb-06), 1446,

1578, 1745, 1749, 2459, 2469, 2822, none

Pump, primary	(SBE 5T, 10500m, 3000RPM)		S/N	1413, 1595, 1596 (30-Dec-04), 2237, 2238,

2825, 2892 (30-Dec-04)

Pump, secondary	(SBE 5T, 10500m, 3000RPM)		S/N	1413, 1595, 1596, 2237, 2238,

2825 (30-Dec-04), 2892 (30-Dec-04), none

Oxygen sensor, primary (SBE 13, SBE 23, SBE 43, 6800m)	S/N	430343, 430345, 430348, 430372 (02-Feb-06p), none

Oxygen sensor, secondary (SBE 13, SBE 23, SBE 43, 6800m)	S/N	430343 (14-Mar-06p), 430345, 430348, 430372, none

Fluorometer, primary	(Sea Tech flashlamp, 3000m,	S/N	117S (30Nov04 10microg/l full scale),

WET Labs ECOFluor, 6000m)		FLRTD-231 (02-Feb-06), none

Fluorometer, secondary	(Sea Tech flashlamp, 3000m)	S/N	117S, FLRTD-231, none

WET Labs ECOFluor, 6000m)

Transmissometer (WET Labs C-Star, 25 cm path, 6000m)	S/N	CST-399DR, CST-480DR (back from factory

08-Mar-05 & 21-Dec-01), CST-593DR, none

Altimeter (Benthos PSA-916, 2500m)			S/N	1017 (new Nov-02), 1075 (ship cal 08-Apr-06,

new 22-Oct-03), none

Light sensor PAR, QSP-200L4S/2300 (Biospherical, 1000m)	S/N	4479, 4701(24-Feb-06), none

Light sensor, Surface SPAR, QSR-2200 (Biospherical)	S/N	20121 (24-Feb-06), 20190, none

Data format (factory & user installed sensors) See also EN425\CTD\CTDReadMe425.txt

Frequency 0		temperature, primary 2107		External voltage 3       	PAR 4701

Frequency 1		conductivity, primary 2469	External voltage 4	DO 0372

Frequency 2		pressure 80016			External voltage 5	DO 0343

Frequency 3		temperature, secondary 2902	Ext voltage 6		spare

Frequency 4		conductivity, secondary 0864	Ext voltage 7		Fluor 231

External voltage 0	Transmissometer 480DR		External voltage 8	spare

External voltage 1	spare				External voltage 9	SPAR 20121

External voltage 2	Alt 1075				Pump1=1596, Pump2=2892

Carousel	 pylon, rosette, levers		GO, SBE	S/N	3210573-0048, 3216020-0175, shiny, dull, none

Frame w/ s/s cage							12 pl SBE alum old/new, 12 pl s/s, 24pl SBE alum, bk1, bk2

Niskins			# on frame: 0 	      1.5, 5, 10, 30 L	Go-Flo, Ext resist OTE / GO, Internal resist, none

Winch / wire						#	1in, 1out, 2, 3, Scanfish / .322”, .500”, .680”, 3/8”, ½”, 9/16”

Slip Rings (Meridian, Focal)				S/N	6.859, 8.538 (10wire), 8.594, Focal 1801308, none

Pinger	(Williams)					S/N	W1, W2, W3, none

(Benthos 2216, 12000m)				S/N	1237, none

Note for SBE 9plus CTD underwater unit S/N 12293-0444, 0523 & 0607:

CTDs 0444, 0523, and 0607 were supplied with an isolated 12-volt power source for a Sea Tech fluorometer in a 3000-meter stainless steel housing. When this fluorometer is used it must be installed on connector JT2 (see drawing 50077). This will put the fluorometer output on A/D channel 1 (External Voltage 1).

- Underwater unit S/N 9P16940-0607, as of x x 2000:

Pressure sensor range						0-10,000 psia, 0-6885 d-Bar

pressure sensor, Digiquartz (with temp comp, SBE)		S/N	80016

Modulo							S/N	MOD12P-439

AD590M (enter in Seacon)					0.012837, , .0128452

AD590B (enter in Seacon)						-9.963864, -8.66958

A/D Input Voltage Range						0 to 5 volts DC

- Logic Board EPROM				Version 1.0

Modem Interface NOT installed

NO Modem Board Microcontroller			Version 2.0A, Version 1.1		P/N N/A, 11124, 11194

Isolated Fluorometer Power Interface NOT Installed