EN425

Pre & Post Cruise:

Check the functionality of the computer program and hand launcher with a bucket of water and a ground wire to the ship. Do a complete test run. Go through each step of the MK12 program. Name the output files that were just created with 9’s so they do not interfere with good data files. Then run the batch file to be sure it works without errors.

Before first cast & with each new assisting scientists/crew:

Review deployment procedures

Safety issues:

Have 2 people available for each XBT drop

Wear your work vest & boots! This is a wet job in rough weather. Even if it’s not rough, always wear your work vest!!!

Ask someone if unsure, especially regarding deck issues

Create the following directories & files for the corresponding purpose &/or clear directories from past cruise:

DOS:	c:\mk12 = Mk12 program files

c:\mk12\dfiles = raw & processed data

c:\mk12\arc425.bat = archives Mk12 XBT files

Each cast:

# Get bottom depth from PTR, Chirp or other reliable source. Do this well before station as well as during cast to be sure of accuracy.

# Get bottom depth from Echosounder. Do this well before station as well as during cast to be sure of accuracy.

Short version for posting:

Check waypoint on Nobeltec Navigation Monitor for ETA & TTG

## Have bridge slow ship to at most 7 knots just before launching XBT

### Have 2 people available for each XBT drop

Acquire new XBT from box

Fill out XBT Logs with Tsal temp, Tsal Salinity, XBT serial # & bottom depth, COG & SOG

On XBT/CTD computer:		Check PC time 		Power up MK21 rack unit

Double click MK21 icon on desktop. This will open the MK12 program

### click the Launch Info button

### Check PC time, GPS position and probe type

### Depth equation should be “Original”

### Options>System menu item and ensure that option Automatically save EDF is checked

### Check realtime data file and backup data file paths

### click the New Drop button

### Options>Launch Information

update Serial # found on top of XBT probe

### Contact bridge for permission to deploy XBT

### Wear work vest when going on deck to launch probe

Monitor should display Spent Canister at bottom

Computer person: ”Remove spent canister”

Open launcher top by turning handle clockwise

Monitor should display Load Probe at bottom

Remove spent XBT & any excess XBT wire

Computer person: “Load probe”

Remove plastic cap from new XBT

Insert new XBT into launcher

Close top of launcher by moving it counter clockwise

Lock XBT into launcher by turning launcher counter clockwise

Monitor should display Launch Probe at bottom

Computer person: “Launch probe”

Pull pin to launch probe.

Monitor should display depth and temp numbers at bottom

Computer person: “Recording and data looks good/bad”

Complete XBT Logs with time, lat, lon and depth of good data.

### Contact bridge when everyone is inside from XBT operations or if another one is going to be done right away due to a faulty probe

On Computer, when the cast has finished:

File, Print

X to close

Leave a spent canister in launcher to protect contact pins.

(Alt) O   for Ok to Export		This creates an ASCII version of the depth vs temp data

### file name should be same as raw file name, except with an edf extension

### F   for File	O    for Open	Use arrow keys to select cast number/file name Enter	O   for Ok

D for Display	C for sCaling, Arrow down to Min Temp=5oC, O , Arrow down to Max Temp=25oC, O

### F   for File	P   to Print

### F   for File	 C   to close file

### F   for File	 Q   for Quit

At c:\mk12\dfilewin  to start windows

Use the icon to copy the XBT files from c:\mk12\dfiles to shipback\XBTbackup

At c:\mk12\dfileCopy T7-000*.*  e:   and  Copy T7-v00*.edf  e:

### F   for File	E   for Export, file name should be same as raw file name, except with an edf extension

D for Display	V for sound Velocity, Enter Salinity from TSal,  (Alt) O for Ok

D for Display	C for sCaling, Arrow down to Min SV=1460 m/S, O , Arrow down to Max SV=1500m/S , O for Ok

### F   for File	E   for Export, file name should be same as raw file name, except with a v after _ and an edf extension

(Alt) O   for Ok		This creates an ASCII version of the depth vs sound velocity data