Launch XBT Using MK21

- Turn on MK21 rack unit and then start WinMK21 program. If the Launch Information window does not pop up, click the Launch Info button. Be sure PC time and the GPS position are correct. Depth equation should be “Original”.

- If probe type is incorrect select the Options>Probe>Selection… menu and select the probe type.

- Select the Options>System menu item and ensure that option Automatically save EDF is checked. Ensure that the realtime data file and backup data file paths are correct.

- Click the New Drop button. After the “MK21 initializing” pop-up window shows itself and closes, the “Realtime Data Acquisition” window will show the current state of the launcher.

- Select the Options>Launch Information menu item. Enter the serial number from the new XBT canister and correct the sequence number if necessary. Remember: the sequence number, and therefore the filename, increments when the XBT drop is initiated. The correct sequence number before you pull the pin is the number of the previous drop.

- Wear a PFD and have someone watch you for the duration of the launch. Open the launcher and remove the spent canister and be sure to clear any old wire from the launcher.  Load a new XBT into the launcher; the program should now indicate that a probe is ready to launch.

- Request permission from bridge to launch XBT.

- For rail mount launcher: Pull pin, return to lab.

- For hand launcher: Stand at leeward (downwind) corner of fantail and with launcher held over the side, pull the pin and allow probe to fall into the water. Get comfortable and wait until probe hits bottom or all wire has spooled out of canister. Break off wire at canister.

- To terminate a cast early press Ctrl-C.

- Select menu item File>Print to print the graph and drop information. This can also be done later. Click the X in the top right corner of the Data Acquisition window to close it.

- Inform bridge when the launch is secured and advise if another launch will be requested in the near future.

- For a new drop go to step 4 above.

Leave a spent canister in launcher to protect contact pins.